NUMERICAL SIMULATION OF PARTIAL SLIP ELASTIC CONTACT

part II: Contact between dissimilar elastic bodies

Sergiu Spinu & Dorin Gradinaru

“Stefan cel Mare” University of Suceava-Romania, Department of Applied Mechanics

Corresponding author: Sergiu Spinu, sergiu.spinu@fim.usv.ro

Abstract: Assumption of uncoupling of normal and tangential effects, present in most contact solvers, is removed in this paper, by employing an iterative numerical procedure in which normal and tangential contact problems are solved successively until convergence is reached. The elastic displacements induced by contact tractions are fully included in the model. The proposed algorithm can be used to accurately simulate the contact between dissimilarly elastic materials. To account for the dissipative nature of friction, the load is applied incrementally, in an outer loop. Numerical simulations predict that, in case of dissimilarly elastic bodies loaded normally, central pressure is underestimated by up to 5% if the effect of tangential friction is neglected.

Key words: numerical simulation, elastic contact, coupled normal and tangential effect, dissimilarly elastic materials.