Study OF safety catch devices of extraction vessels using CAD/CAE software

Iosif Dumitrescu 1, Vilhelm Itu1, Bogdan Cozma1

1University of Petroşani-Romania, Department of Mechanical Engineering, Industrial and Transport

Corresponding author: Dumitrescu Iosif, iosif_dumi@yahoo.com

Abstract: The safety catch device are safety devices, that eliminates extraction vessels to fall in mine shaft if the cable breaks or the connecting elements (rods, a joining, etc.). Their use is mandatory. In this paper, using software, CAD / CAE, are studying the conditions to be fulfilled by the safety catch devices: to operate safe; smoothly and soon disappeared from the cable tension or decreased by more than 40% by weight of the extraction vessel; map of the extraction area not exceeding 100 mm in the moment of opening the safety catch device; have simple construction; low weight and higher safety factor of 8; according to safety rules.

Key words: safety catch device, extraction vessels, safe operation, CAD/CAE software.